# QuickCell-12 Parameter Guide
2-Input & 3-Input Gate Reference

command-line parameter reference for filter calibration

This document lists references for all the tunable parameters in the QuickCell-12 prototype. This includes both the 2- input gate design (d = 3 canvas SiDBs, μ₋ = −0.32 eV) and 3 input gate designs (d = 4 canvas SiDBs, μ₋ = −0.31 eV) and  each tunable parameter corresponds to ( Pruning filter, increasing/decreasing effect, and useful range )

A number of tunable parameters in the prototype represent physical margins, while the rest of them refer to robustness parameters.

There are several calibrated prunning margins that help filters to work either in the permissive physical feasibility mode or in the restrictive robustness-oriented mode. smaller or larger value does not necessarily imply anything better. Instead, such parameters define the tradeoff between the candidate set size and pruning aggressiveness. Permissive modes should be applied for 2-input gates due to their small canvas and geometrical constraints, but 3-input gates support more aggressive filter settings

# Gate Overview

## 2-Input Gates (15 gates)

| Category | Gate Names | Inputs | Outputs | d | μ₋ [eV] | Script |
| --- | --- | --- | --- | --- | --- | --- |
| 2-in, 1-out | AND, NAND, OR, NOR, XOR, XNOR, LT, GT, LE, GE | 2 | 1 | 3 | −0.32 | run_gate_report_2input.py |
| 2-in, 2-out | DOUBLE_WIRE, CX, HA | 2 | 2 | 3 | −0.32 | run_gate_report_2input.py |
| 1-in, 1-out | WIRE, INV | 1 | 1 | 3 | −0.32 | run_gate_report_2input.py |

## 3-Input Gates (10 gates)

| Category | Gate Names | Inputs | Outputs | d | μ₋ [eV] | Script |
| --- | --- | --- | --- | --- | --- | --- |
| 3-in, 1-out | AND3, XOR-AND, OR-AND, ONEHOT, MAJ, GAMBLE, DOT, ITE, AND-XOR, XOR3 | 3 | 1 | 4 | −0.31 | run_gate_report.py |

# Key Differences Between 2-Input and 3-Input Setups

| Parameter | 2-Input Gates | 3-Input Gates |
| --- | --- | --- |
| μ₋ (mu_minus) | −0.32 eV | −0.31 eV |
| Canvas SiDBs (d) | 3 | 4 |
| Input patterns | 4 (2²) or 2 (1¹) | 8 (2³) |
| Skeleton (1-out) | 17 SiDBs | 25 SiDBs |
| Canvas size (1-out) | 99 positions | 143 positions |
| ¦Ld¦ (1-out) | C(99,3) = 156,849 | C(143,4) = 17,058,156 |
| Total gates | 15 | 10 |
| Report script | run_gate_report_2input.py | run_gate_report.py |
| Export script | export_siqad_validation_2input.py | export_siqad_validation.py |
| Results folder | results_2input/ | results/ |

# Example Commands — 3-Input Gates

## Balanced Setting (recommended for paper)

python run_gate_report.py --gate AND3 --samples 100000 --workers 4 --chunksize 2000 --strict-profile balanced --io-margin 0.0001 --connectivity-radius 9.0 --d-min 3.1 --positive-charge-margin 0.02 --charge-min-fraction 0.25 --charge-max-fraction 1.0 --charge-count-conflict-radius 4.3 --input-disturbance 0.001 --pressure-margin 0.001 --energy-margin 0.01 --f11-mode strict-hop --f11-min-support-states 2

## Run ALL 3-input gates

python run_gate_report.py --gate ALL --samples 100000 --workers 4 --chunksize 2000 --strict-profile balanced --io-margin 0.0001

## Export SiQAD files — 3-input

python export_siqad_validation.py --gate ALL --samples 100000 --workers 4 --chunksize 2000 --strict-profile balanced --io-margin 0.0001 --max-candidates 50

# Example Commands — 2-Input Gates

## Balanced Setting (easy gates)

python run_gate_report_2input.py --gate AND --samples 200000 --workers 4 --strict-profile safe --io-margin 0.02

## Run ALL 2-input gates (maximum success)

python run_gate_report_2input.py --gate ALL --samples 1000000 --workers 4 --chunksize 5000 --canvas-d 3 --seed 12345 --sim-time 0.01 --strict-profile safe --io-margin 0.05 --pressure-margin 0.0 --d-min 1.0 --positive-charge-margin 0.0 --charge-min-fraction 0.0 --charge-max-fraction 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --energy-margin 1.0 --f11-mode counted --f11-min-support-states 1 --selection-target 20 --selection-min-margin 0.0

## Difficult gates (XOR, XNOR, CX, HA)

python run_gate_report_2input.py --gate XOR --samples 1000000 --workers 4 --strict-profile safe --io-margin 0.05 --d-min 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --f11-mode counted

## Export SiQAD files — ALL 2-input gates

python export_siqad_validation_2input.py --gate ALL --samples 1000000 --workers 4 --chunksize 5000 --max-candidates 20 --canvas-d 3 --strict-profile safe --io-margin 0.05 --d-min 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --energy-margin 1.0 --f11-mode counted --selection-target 20 --selection-min-margin 0.0

## Export individual 2-input gates

AND / NAND / OR / NOR / LE / GE / WIRE / INV (easy)

python export_siqad_validation_2input.py --gate AND --samples 200000 --workers 4 --max-candidates 20 --strict-profile safe --io-margin 0.02

XOR (hardest 2-input gate)

python export_siqad_validation_2input.py --gate XOR --samples 1000000 --workers 4 --max-candidates 20 --strict-profile safe --io-margin 0.05 --d-min 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --f11-mode counted

XNOR

python export_siqad_validation_2input.py --gate XNOR --samples 1000000 --workers 4 --max-candidates 20 --strict-profile safe --io-margin 0.05 --d-min 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --f11-mode counted

LT (A < B)

python export_siqad_validation_2input.py --gate LT --samples 500000 --workers 4 --max-candidates 20 --strict-profile safe --io-margin 0.05 --d-min 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --f11-mode counted

GT (A > B)

python export_siqad_validation_2input.py --gate GT --samples 500000 --workers 4 --max-candidates 20 --strict-profile safe --io-margin 0.05 --d-min 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --f11-mode counted

DOUBLE_WIRE (2-in, 2-out)

python export_siqad_validation_2input.py --gate DOUBLE_WIRE --samples 500000 --workers 4 --max-candidates 20 --strict-profile safe --io-margin 0.05 --d-min 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --f11-mode counted

CX — Crossover (2-in, 2-out)

python export_siqad_validation_2input.py --gate CX --samples 1000000 --workers 4 --max-candidates 20 --strict-profile safe --io-margin 0.05 --d-min 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --f11-mode counted

HA — Half Adder (2-in, 2-out)

python export_siqad_validation_2input.py --gate HA --samples 1000000 --workers 4 --max-candidates 20 --strict-profile safe --io-margin 0.05 --d-min 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --f11-mode counted

## Analyze SiQAD results

# 3-input gates
python analyze_siqad_results.py --root validation_exports --all
# 2-input gates
python analyze_siqad_results.py --root validation_exports_2input --all

# Complete Parameter Summary Table

this table lists every tunable parameters ( the filter it controls, meaning, effect of smaller/larger values, recommended ranges for both 2-input and 3-input gate setups)

| Parameter | Filter | Meaning | Smaller Value | Larger Value | Range (3-in) | Range (2-in) | Current (3-in) | Current (2-in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| --d-min | F1 | Minimum distance between canvas SiDBs | Weaker F1 | Stricter F1 | 3.0 – 4.5 | 1.0 – 4.5 | 3.1 | 1.0 |
| --positive-charge-margin | F4 | Safety margin for positive-charge activation | Weaker F4 | Stricter F4 | 0.005 – 0.05 | 0.0 – 0.05 | 0.02 | 0.0 |
| --charge-min-fraction | F5 | Minimum required negative-charge fraction | Weaker F5 | Stricter F5 | 0.20 – 0.28 | 0.0 – 0.25 | 0.25 | 0.0 |
| --charge-max-fraction | F5 | Maximum allowed negative-charge fraction | Stricter if reduced | Weaker if increased | 0.8 – 1.0 | 0.8 – 1.0 | 1.0 | 1.0 |
| --charge-count-conflict-radius | F5 | Conflict radius for simultaneous negative canvas occupation | Weaker F5 | Stricter F5 | 4.0 – 6.5 | None – 6.5 | 4.3 | None |
| --input-disturbance | F6 | Maximum canvas-induced disturbance on input pins | Stricter F6 | Weaker F6 | 0.001 – 0.01 | 0.3 – 1.0 | 0.001 | 1.0 |
| --connectivity-radius | F7 | Electrostatic connectivity radius for path check | Stricter F7 | Weaker F7 | 8.5 – 9.8 | 15.0 – 25.0 | 9.0 | 25.0 |
| --pressure-margin | F9 | Required safety margin for output BDL pressure | Weaker F9 | Stricter F9 | 0.0001 – 0.002 | 0.0 | 0.001 | 0.0 |
| --energy-margin | F10 | Energy margin for relaxed energetic pruning | Stricter F10 | Weaker F10 | 0.005 – 0.05 | 0.5 – 1.0 | 0.01 | 1.0 |
| --f11-mode | F11 | Physical infeasibility execution mode | N/A | N/A | counted / strict-pop / strict-hop | counted / strict-pop / strict-hop | strict-hop | counted |
| --f11-min-support-states | F11 | Min metastable supporting states per input | Weaker F11 | Stricter F11 | 1 – 3 | 1 | 2 | 1 |
| --io-margin | F12 | Energy tolerance for I/O instability pruning | Stricter F12 | Weaker F12 | 0.00001 – 0.002 | 0.02 – 0.05 | 0.0001 | 0.05 |
| --strict-profile | Multiple | Preset profile: safe/balanced/strict | N/A | N/A | Safe, balanced , strict | safe ,balanced ,strict | balanced | safe |
| --samples | N/A | Number of random layout samples to evaluate | Faster but may miss candidates | Slower but more thorough | 100,000 – 500,000 | 200,000 – 1,000,000 | 100,000 | 1,000,000 |
| --canvas-d | N/A | Number of canvas SiDBs per layout | N/A | N/A | 4 (fixed) | 3 (fixed) | 4 | 3 |
| --workers | N/A | Number of parallel worker processes | N/A | N/A | 1 – 8 | 1 – 8 | 4 | 4 |
| --chunksize | N/A | Layouts per work chunk | N/A | N/A | 1000 – 5000 | 1000 – 5000 | 2000 | 5000 |
| --seed | N/A | Random seed for reproducibility | N/A | N/A | Any integer | Any integer | 12345 | 12345 |
| --sim-time | N/A | Assumed per-layout simulation time for speedup calc | N/A | N/A | 0.01 | 0.01 | 0.01 | 0.01 |
| --selection-target | Post-F12 | Top-K energy-margin ranking after F12 | N/A | N/A | 0 (disabled) or 20–50 | 0 or 20–50 | 0 | 20 |
| --selection-min-margin | Post-F12 | Minimum energy margin for exported candidates | N/A | N/A | None or 0.001 | None or 0.0 | None | 0.0 |

# Filter-by-Filter Parameter Explanation

## F1 — Minimum Distance Pruning

--d-min  : 
F1 remove a layout , if at least two selected canvas SiDBs are closer than d_min. in the rent paper like canvas, the grid spacing is approximately 3.0 nm. For 3-input gates, using --d-min 3.1 introduces a design rule spacing threshold. For 2-input gates with difficult truth tables, using --d-min 1.0 allows more layouts to survive and increases the chance of finding valid candidates

| Value | Expected Effect | Recommended For |
| --- | --- | --- |
| 1.0 | Very permissive , almost no pruning | 2-input difficult gates (XOR, CX, HA) |
| 3.1 | Removes immediate neighboring placements | 3-input gates (standard) |
| 4.3 | Also removes diagonal neihbors (√(3²+3²) ≈ 4.24) | 3-input aggressive |
| > 5.0 | Very strict , may remove most layouts | Rarely useful |

## F4 — Positive Charge Pruning

--positive-charge-margin  : 
F4 removes layouts that can enter a positive charge regime ,  A margin of 0.0 gives the original behavior and  for 2-input gates that struggle to find candidates, keep this at 0.0

| Value | Expected Effect |
| --- | --- |
| 0.0 | Original F4 behavior (recommended for 2-input) |
| 0.005 | Mild additional pruning |
| 0.01 – 0.02 | Moderate pruning (recommended for 3-input balanced) |
| 0.05 | Strong pruning; possible over-pruning |

## F5 — Charge Count Bound Pruning

F5 uses charge capacity constraints. Three sub parameters control it:
 --charge-min-fraction: minimum required negative charge fraction
 --charge-max-fraction: maximum allowed negative charge fraction
 --charge-count-conflict-radius: conflict distance for simultaneous negative occupation

| Parameter | 3-Input Range | 2-Input Range |
| --- | --- | --- |
| --charge-min-fraction | 0.20 – 0.28 | 0.0 (permissive) |
| --charge-max-fraction | 0.8 – 1.0 | 1.0 |
| --charge-count-conflict-radius | 4.0 – 6.5 | None (disabled) |

## F6 — Input Pin Disturbance Pruning

--input-disturbance  : 
F6 protects input BDL pins from excessive electrostatic back action , for 2-input difficult gates, a very large value (e.g 1.0) effectively disables this filter to preserve candidates

| Value | Expected Effect |
| --- | --- |
| None | Uses default threshold; may prune little |
| 1.0 | Effectively disables F6 (recommended for 2-input difficult) |
| 0.01 | Mild pruning |
| 0.003 | Moderate pruning |
| 0.001 | Very strict (recommended for 3-input balanced) |

## F7 — Electrostatic Path Connectivity Pruning

--connectivity-radius   :
F7 constructs a connectivity graph,  the layout is rejected if input/output reachability is not satisfied within the radius. for 2-input gates, a large radius (25.0) effectively loosens this filter

| Value | Context | Expected Effect |
| --- | --- | --- |
| 5.0 | Any | It is too strict , removes almost everything |
| 8.5 – 9.0 | 3-input | Useful pruning region |
| 9.5 – 10.0 | 3-input | Milder pruning |
| 15.0 – 25.0 | 2-input | Permissive, preserves candidates |
| 25.0 | 2-input difficult | Effectively minimal pruning |

## F8 — Output Potential Bound Pruning

F8 checks whether the output BDL sites can sustain the required charge states given the potential interval from skeleton + canvas , this filter has no explicit tunable margin parameter ,   it uses the physical charge  stability windows from μ₋ and μ₊

## F9 — Output Pressure Pruning

--pressure-margin : 
F9 checks whether the output BDL pair can be biased in the required direction,  for 2-input gates, keeping this at 0.0 is recommended

| Value | Expected Effect |
| --- | --- |
| 0.0 | Basic sign check only (recommended for 2-input) |
| 0.0005 | Moderate pruning |
| 0.001 | Stronger pruning (recommended for 3-input balanced) |
| 0.002 | Very strict , may remove many candidates |

## F10 — Relaxed Energy Pruning

--energy-margin :
F10 compares relaxed minimum energies,  If an incorrect assignment is lower by more than energy margin, the layout will removed
 Smaller = stricter, Larger = weaker
For 2 input difficult gates, a large value (1.0) effectively disables F10

| Value | Expected Effect |
| --- | --- |
| 1.0 | It is very weak, almost no pruning (2-input difficult) |
| 0.05 | Weak( F10) |
| 0.01 | Balanced (3-input recommended) |
| 0.005 | Strong pruning |
| 0.001 | Very strict |

## F11 — Physical Infeasibility Pruning

--f11-mode and --f11-min-support-states
F11 checked whether the correct I/O can be physically supported by at least one metastable canvas charge configuration

| --f11-mode | Meaning | Recommended For |
| --- | --- | --- |
| counted | F11 is not executed , counted as passed | 2 input gates (maximize candidates) |
| strict-pop | Population stability is checked | 3-input conservative |
| strict-hop | Population + single-electron hop stabilitey | 3 input balanced/aggressive |

| --f11-min-support-states | Expected Effect | Recommended |
| --- | --- | --- |
| 1 | Standard feasibility | 2-input gates, 3-input conservative |
| 2 | More robust | 3 input balanced |
| 3 | Stronger robustness | 3 input aggressive |
| > 4 | May be too strict for d = 3 or d = 4 | Rarely useful |

## F12 — I/O Signal Instability Pruning

--io-margin
F12 is the final and most critical filter. this checks whether an incorrect output assignment has lower energy than the correct one
smaller = stricter, larger = weaker.
this is most important parameter for getting candidates,  If no candidates are found, increase this value first

| Value | Expected Effect | Recommended For |
| --- | --- | --- |
| 0.05 | Very permissive | 2 input difficult gates (XOR, CX, HA) |
| 0.02 | Permissive | 2 input easy gates |
| 0.001 | Moderate | 3-input conservative |
| 0.0001 | Strict | 3 input balanced |
| 0.00001 | Very strict | 3 input aggressive |

# Recommended Parameter Sets

## 3-Input Gates

### Conservative / Candidate-Preserving (3-input)

--strict-profile balanced --d-min 3.1 --positive-charge-margin 0.01 --charge-min-fraction 0.20 --charge-max-fraction 1.0 --charge-count-conflict-radius 4.3 --input-disturbance 0.003 --connectivity-radius 9.5 --pressure-margin 0.0005 --energy-margin 0.02 --f11-mode strict-pop --f11-min-support-states 1 --io-margin 0.0005

### Balanced (3-input) — Recommended for paper

--strict-profile balanced --d-min 3.1 --positive-charge-margin 0.02 --charge-min-fraction 0.25 --charge-max-fraction 1.0 --charge-count-conflict-radius 4.3 --input-disturbance 0.001 --connectivity-radius 9.0 --pressure-margin 0.001 --energy-margin 0.01 --f11-mode strict-hop --f11-min-support-states 2 --io-margin 0.0001

### Aggressive Pruning (3-input)

--strict-profile balanced --d-min 4.3 --positive-charge-margin 0.03 --charge-min-fraction 0.27 --charge-max-fraction 1.0 --charge-count-conflict-radius 6.1 --input-disturbance 0.0008 --connectivity-radius 8.7 --pressure-margin 0.002 --energy-margin 0.005 --f11-mode strict-hop --f11-min-support-states 3 --io-margin 0.00005

## 2-Input Gates

### Easy gates (AND, OR, NAND, NOR, LE, GE, WIRE, INV)

--strict-profile safe --samples 200000 --io-margin 0.02 --d-min 1.0 --positive-charge-margin 0.0 --charge-min-fraction 0.0 --charge-max-fraction 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --pressure-margin 0.0 --energy-margin 1.0 --f11-mode counted --f11-min-support-states 1

### Difficult gates (XOR, XNOR, LT, GT, DOUBLE_WIRE, CX, HA)

--strict-profile safe --samples 1000000 --io-margin 0.05 --d-min 1.0 --positive-charge-margin 0.0 --charge-min-fraction 0.0 --charge-max-fraction 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --pressure-margin 0.0 --energy-margin 1.0 --f11-mode counted --f11-min-support-states 1

### Maximum success (ALL 2-input gates)

--strict-profile safe --samples 1000000 --chunksize 5000 --canvas-d 3 --seed 12345 --io-margin 0.05 --pressure-margin 0.0 --d-min 1.0 --positive-charge-margin 0.0 --charge-min-fraction 0.0 --charge-max-fraction 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --energy-margin 1.0 --f11-mode counted --f11-min-support-states 1 --selection-target 20 --selection-min-margin 0.0

# Troubleshooting: No Candidates Found

If a gate produce zero candidates, follow these steps in order:

| Step | Action | Why |
| --- | --- | --- |
| 1 | Increase ( --io-margin to 0.05 ) | F12 is the strictest filter |
| 2 | Change ( --strict-profile to safe ) | Relaxes wire_radius and d_min |
| 3 | Increase (--samples to 1000000 ) | More layouts explored |
| 4 | Set (--f11-mode counted ) | Disables strict F11 |
| 5 | Set (--input-disturbance 1.0) | Effectively disables F6 |
| 6 | Set (--connectivity-radius 25.0 ) | Effectively disables F7 |
| 7 | Set (--energy-margin 1.0 ) | Effectively disables F10 |
| 8 | Set ( --d-min 1.0 ) | Effectively disables F1 |
| 9 | Set (--positive-charge-margin 0.0 ) | Original F4 behavior |

If no candidates are found, the first parameter to relax is always --io-margin, followed by --strict-profile safe, then --samples increase. The remaining filter parameters should only be relaxed if the first three steps are insufficient

# Gate Difficulty Classification

## 2-Input Gates

| Difficulty | Gates | Recommended --samples | Recommended --io-margin |
| --- | --- | --- | --- |
| Easy | AND, NAND, OR, NOR, LE, GE, WIRE, INV | 200,000 | 0.02 |
| Medium | LT, GT, DOUBLE_WIRE | 500,000 | 0.05 |
| Hard | XOR, XNOR, CX, HA | 1,000,000 | 0.05 |

## 3-Input Gates

| Difficulty | Gates | Recommended --samples | Recommended --io-margin |
| --- | --- | --- | --- |
| Easy | AND3, OR-AND, MAJ | 100,000 | 0.0001 |
| Medium | XOR-AND, ONEHOT, GAMBLE, ITE | 100,000 | 0.0001 |
| Hard | DOT, AND-XOR, XOR3 | 200,000 | 0.0005 |

# Filter Type Classification

| Filter | Type | Tunable Parameter |
| --- | --- | --- |
| F1 | Design-rule / geometric pruning | --d-min |
| F2 | Symmetry-based canonical pruning | (automatic) |
| F3 | Wire interference geometric pruning | --wire-radius (via profile) |
| F4 | Physical positive-charge guard | --positive-charge-margin |
| F5 | Heuristic charge-capacity pruning | --charge-min/max-fraction, --charge-count-conflict-radius |
| F6 | Heuristic input-integrity pruning | --input-disturbance |
| F7 | Heuristic electrostatic reachability | --connectivity-radius |
| F8 | Analytical output-potential feasibility | (no explicit margin) |
| F9 | Analytical output-pressure feasibility | --pressure-margin |
| F10 | Relaxed energetic pre-pruning | --energy-margin |
| F11 | Enumerative physical feasibility | --f11-mode, --f11-min-support-states |
| F12 | Enumerative I/O energetic stability | --io-margin |

# Complete Command Reference

## Full 3-input report command

python run_gate_report.py --gate ALL --samples 100000 --workers 4 --chunksize 2000 --seed 12345 --sim-time 0.01 --strict-profile balanced --io-margin 0.0001 --pressure-margin 0.001 --connectivity-radius 9.0 --d-min 3.1 --positive-charge-margin 0.02 --charge-min-fraction 0.25 --charge-max-fraction 1.0 --charge-count-conflict-radius 4.3 --input-disturbance 0.001 --energy-margin 0.01 --f11-mode strict-hop --f11-min-support-states 2 --selection-target 0

## Full 2-input report command

python run_gate_report_2input.py --gate ALL --samples 1000000 --workers 4 --chunksize 5000 --canvas-d 3 --seed 12345 --sim-time 0.01 --strict-profile safe --io-margin 0.05 --pressure-margin 0.0 --connectivity-radius 25.0 --d-min 1.0 --positive-charge-margin 0.0 --charge-min-fraction 0.0 --charge-max-fraction 1.0 --input-disturbance 1.0 --energy-margin 1.0 --f11-mode counted --f11-min-support-states 1 --selection-target 20 --selection-min-margin 0.0

## Full 3-input SiQAD export command

python export_siqad_validation.py --gate ALL --samples 100000 --workers 4 --chunksize 2000 --max-candidates 50 --strict-profile balanced --io-margin 0.0001

## Full 2-input SiQAD export command

python export_siqad_validation_2input.py --gate ALL --samples 1000000 --workers 4 --chunksize 5000 --max-candidates 20 --canvas-d 3 --strict-profile safe --io-margin 0.05 --d-min 1.0 --input-disturbance 1.0 --connectivity-radius 25.0 --energy-margin 1.0 --f11-mode counted --selection-target 20 --selection-min-margin 0.0

# Truth Tables Reference

## 2-Input, 1-Output Gates

| Gate | Hex | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- | --- |
| AND | 0x8 | 0 | 0 | 0 | 1 |
| NAND | 0x7 | 1 | 1 | 1 | 0 |
| OR | 0xE | 0 | 1 | 1 | 1 |
| NOR | 0x1 | 1 | 0 | 0 | 0 |
| XOR | 0x6 | 0 | 1 | 1 | 0 |
| XNOR | 0x9 | 1 | 0 | 0 | 1 |
| LT | 0x2 | 0 | 1 | 0 | 0 |
| GT | 0x4 | 0 | 0 | 1 | 0 |
| LE | 0xB | 1 | 1 | 0 | 1 |
| GE | 0xD | 1 | 0 | 1 | 1 |

## 2-Input, 2-Output Gates

| Gate | Hex (O₁,O₂) | 00→ | 01→ | 10→ | 11→ |
| --- | --- | --- | --- | --- | --- |
| DOUBLE_WIRE | (0xC, 0xA) | 0,0 | 0,1 | 1,0 | 1,1 |
| CX | (0xA, 0xC) | 0,0 | 1,0 | 0,1 | 1,1 |
| HA | (0x6, 0x8) | 0,0 | 1,0 | 1,0 | 0,1 |

## 1-Input, 1-Output Gates

| Gate | Hex | 0 | 1 |
| --- | --- | --- | --- |
| WIRE | 0x2 | 0 | 1 |
| INV | 0x1 | 1 | 0 |

## 3-Input, 1-Output Gates

| Gate | Hex | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AND3 | 0x80 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| XOR-AND | 0x28 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| OR-AND | 0xA8 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |
| ONEHOT | 0x16 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |
| MAJ | 0xE8 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |
| GAMBLE | 0x81 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| DOT | 0x52 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| ITE | 0xD8 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |
| AND-XOR | 0x6A | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |
| XOR3 | 0x96 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |